## Parameters

The following table lists the data used in Flight Simulator.  Examples of these can be found in the XML-based aircraft gauge files.

Aircraft Engine Data

Aircraft Fuel Data

Aircraft L ights Data

Aircraft Position and Speed Data

Aircraft Flight Instrumentation Data

Aircraft Avionics Data

Aircraft Controls Data

Aircraft Autopilot Data

Aircraft Environment Data

Helicopter Specific Data

Aircraft Systems Data

Aircraft Miscellaneous Data

Aircraft String Data

Simulator Global Data

Aircraft Data

Engine Data

Note: Engine parameters take an index (1,2,…) to determine which engine is being queried.

| Simulation Variable | Description |
| --- | --- |
| NUMBER OF ENGINES | Self explanatory |
| GENERAL ENG COMBUSTION | Combustion flag (bool) |
| GENERAL ENG MASTER ALTERNATOR | Alternator (generator) switch (bool) |
| GENERAL ENG FUEL PUMP SWITCH | Fuel pump switch (bool) |
| GENERAL ENG FUEL PUMP ON | Fuel pump on/off (bool) |
| GENERAL ENG RPM | Engine rpm |
| GENERAL ENG PCT MAX RPM | Percent of max rated rpm |
| GENERAL ENG MAX REACHED RPM | Maximum attained rpm |
| GENERAL ENG THROTTLE LEVER POSITION | Percent of max throttle position |
| GENERAL ENG MIXTURE LEVER POSITION | Percent of max mixture lever position |
| GENERAL ENG PROPELLER LEVER POSITION | Percent of max prop lever position |
| GENERAL ENG STARTER | Engine starter on/off (bool) |
| GENERAL ENG EXHAUST GAS TEMPERATURE | Self explanatory |
| GENERAL ENG OIL PRESSURE | Self explanatory |
| GENERAL ENG OIL LEAKED PERCENT | Percent of max oil capacity leaked |
| GENERAL ENG COMBUSTION SOUND PERCENT | Percent of maximum engine sound |
| GENERAL ENG DAMAGE PERCENT | Percent of total engine damage |
| GENERAL ENG OIL TEMPERATURE | Self explanatory |
| GENERAL ENG FAILED | Fail flag (bool) |
| GENERAL ENG GENERATOR SWITCH | Alternator (generator) switch (bool) |
| GENERAL ENG GENERATOR ACTIVE | Alternator (generator) on/off (bool) |
| GENERAL ENG ANTI ICE POSITION | Engine anti-ice switch (bool) |
| GENERAL ENG FUEL VALVE | Fuel valve state (bool) |
| GENERAL ENG FUEL PRESSURE | Self Explanatory |
| GENERAL ENG ELAPSED TIME | Total engine elapsed time |
| RECIP ENG COWL FLAP POSITION | Percent cowl flap opened |
| RECIP ENG PRIMER | Engine primer position |
| RECIP ENG MANIFOLD PRESSURE | Self explanatory |
| RECIP ENG ALTERNATE AIR POSITION | Alternate air control |
| RECIP ENG COOLANT RESERVOIR PERCENT | Percent coolant available |
| RECIP ENG LEFT MAGNETO | Left magneto state (bool) |
| RECIP ENG LEFT MAGNETO | Right magneto state (bool) |
| RECIP ENG BRAKE POWER | Brake power produced by engine |
| RECIP ENG STARTER TORQUE | Torque produced by engine |
| RECIP ENG TURBOCHARGER FAILED | Turbo failed state (bool) |
| RECIP ENG EMERGENCY BOOST ACTIVE | War emergency power active (bool) |
| RECIP ENG EMERGENCY BOOST ELAPSED TIME | Elapsed time war emergency power active |
| RECIP ENG WASTEGATE POSITION | Percent turbo wastegate closed |
| RECIP ENG TURBINE INLET TEMPERATURE | Self explanatory |
| RECIP ENG CYLINDER HEAD TEMPERATURE | Self explanatory |
| RECIP ENG RADIATOR TEMPERATURE | Self explanatory |
| RECIP ENG FUEL AVAILABLE | Self explanatory |
| RECIP ENG FUEL FLOW | Self explanatory |
| RECIP ENG FUEL TANK SELECTOR | Fuel tank selected for engine (enum: See tank list below) |
| RECIP ENG FUEL TANKS USED | Fuel tanks used (bit flags) |
| RECIP ENG FUEL NUMBER TANKS USED | Number of tanks currently being used |
| RECIP CARBURETOR TEMPERATURE | Self explanatory |
| RECIP MIXTURE RATIO | Fuel / Air mixture ratio |
| TURB ENG N1 | Self explanatory |
| TURB ENG N2 | Self explanatory |
| TURB ENG CORRECTED N1 | Self explanatory |
| TURB ENG CORRECTED N2 | Self explanatory |
| TURB ENG CORRECTED FF | Corrected fuel flow |
| TURB ENG MAX TORQUE PERCENT | Percent of max rated torque |
| TURB ENG PRESSURE RATIO | Self explanatory |
| TURB ENG ITT | Self explanatory |
| TURB ENG AFTERBURNER | Afterburner state (bool) |
| TURB ENG JET THRUST | Self explanatory |
| TURB ENG BLEED AIR | Bleed air pressure |
| TURB ENG TANK SELECTOR | Fuel tank selected for engine (enum: See tank list below) |
| TURB ENG TANKS USED | Fuel tanks used (bit flags) |
| TURB ENG NUM TANKS USED | Number of tanks currently being used |
| TURB ENG FUEL FLOW PPH | Self explanatory |
| TURB ENG FUEL AVAILABLE | Self explanatory |
| TURB ENG REVERSE NOZZLE PERCENT | Percent thrust reverser nozzles deployed |
| TURB ENG VIBRATION | Engine vibration value |
| ENG FAILED | Failure flag (bool) |
| ENG RPM ANIMATION PERCENT | Percent max rated rpm – used for visual animation |
| ENG ON FIRE | On fire state (bool) |
| ENG FUEL FLOW BUG POSITION | Fuel flow reference |
| ENGINE CONTROL SELECT | “Selected” engines (bit flags) |
| THROTTLE LOWER LIMIT | Percent throttle defining lower limit (negative for reverse thrust equipped airplanes) |
| ENGINE TYPE | Engine type (enum: 0=Piston, 1=Jet, 2=None, 3=Helo(Bell) turbine, 4=Unsupported, 5=Turboprop) |
| MASTER IGNITION SWITCH | Aircraft “master” ignition switch (grounds all engines’ magnetos) |
| PROP RPM | Self explanatory |
| PROP MAX RPM PERCENT | Percent of max rated rpm |
| PROP THRUST | Self explanatory |
| PROP BETA | Prop blade pitch angle |
| PROP FEATHERING INHIBIT | Feathering inhibit flag (bool) |
| PROP FEATHERED | Feathered state (bool) |
| PROP SYNC DELTA LEVER | Corrected prop correction input on “slaved” engine |
| PROP AUTO FEATHER ARMED | Auto-feather armed state (bool) |
| PROP FEATHER SWITCH | Prop feather switch (bool) |
| PANEL AUTO FEATHER SWITCH | Auto-feather arming switch (bool) |
| PROP SYNC ACTIVE | Self explanatory |
| PROP DEICE SWITCH | Self explanatory |

Tank/Selector enumerated values:

0  = Off

1  = All

2  = Left

3  = Right

4  = Left_aux

5  = Right_aux

6  = Center

7  = Center2

8  = Center3

9  = External1

10 = External2

11 = Right_tip

12 = Left_tip

13 = Crossfeed

14 = Crossfeed left to right

15 = Crossfeed right to left

16 = Both

17 = External

18 = Isolate

Fuel Data

| Simulation Variable | Description |
| --- | --- |
| FUEL TANK CENTER LEVEL | Percent of maximum capacity |
| FUEL TANK CENTER2 LEVEL | Percent of maximum capacity |
| FUEL TANK CENTER3 LEVEL | Percent of maximum capacity |
| FUEL TANK LEFT MAIN LEVEL | Percent of maximum capacity |
| FUEL TANK LEFT AUX LEVEL | Percent of maximum capacity |
| FUEL TANK LEFT TIP LEVEL | Percent of maximum capacity |
| FUEL TANK RIGHT MAIN LEVEL | Percent of maximum capacity |
| FUEL TANK RIGHT AUX LEVEL | Percent of maximum capacity |
| FUEL TANK RIGHT TIP LEVEL | Percent of maximum capacity |
| FUEL TANK EXTERNAL1 LEVEL | Percent of maximum capacity |
| FUEL TANK EXTERNAL2 LEVEL | Percent of maximum capacity |
| FUEL TANK CENTER CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK CENTER2 CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK CENTER3 CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK LEFT MAIN CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK LEFT AUX CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK LEFT TIP CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK RIGHT MAIN CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK RIGHT AUX CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK RIGHT TIP CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK EXTERNAL1 CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK EXTERNAL2 CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL LEFT CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL RIGHT CAPACITY | Maximum capacity in volume (e.g. gallons) |
| FUEL TANK CENTER QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK CENTER2 QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK CENTER3 QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK LEFT MAIN QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK LEFT AUX QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK LEFT TIP QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK RIGHT MAIN QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK RIGHT AUX QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK RIGHT TIP QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK EXTERNAL1 QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TANK EXTERNAL2 QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL LEFT QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL RIGHT QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL TOTAL QUANTITY | Current quantity in volume (e.g. gallons) |
| FUEL WEIGHT PER GALLON | Self explanatory |
| FUEL TANK SELECTOR | Which tank is selected (enum: See tank list above) (include engine index) |
| FUEL CROSS FEED | Cross feed valve (enum: 0=Closed, 1=Open) |
| FUEL TOTAL CAPACITY | Total capacity of the aircraft |
| FUEL SELECTED QUANTITY PERCENT | Percent or capacity for selected tank |
| FUEL SELECTED QUANTITY | Quantity of selected tank |
| FUEL TOTAL QUANTITY WEIGHT | Current total fuel weight of the aircraft |
| NUM FUEL SELECTORS | Number of selectors on the aircraft |
| UNLIMITED FUEL | Unlimited fuel flag (bool) |
| ESTIMATED FUEL FLOW | Estimated fuel flow at cruise |

Lights Data

| Simulation Variable | Description |
| --- | --- |
| LIGHT STROBE | Light switch state (bool) |
| LIGHT PANEL | Light switch state (bool) |
| LIGHT LANDING | Light switch state (bool) |
| LIGHT TAXI | Light switch state (bool) |
| LIGHT BEACON | Light switch state (bool) |
| LIGHT NAV | Light switch state (bool) |
| LIGHT LOGO | Light switch state (bool) |
| LIGHT WING | Light switch state (bool) |
| LIGHT RECOGNITION | Light switch state (bool) |
| LIGHT CABIN | Light switch state (bool) |

Position and Speed Data

| Simulation Variable | Description |
| --- | --- |
| GROUND VELOCITY | Speed relative to the earth’s surface |
| TOTAL WORLD VELOCITY | Speed relative to the earth’s center |
| VELOCITY BODY Z | True longitudinal speed, relative to aircraft axis |
| VELOCITY BODY X | True lateral speed, relative to aircraft axis |
| VELOCITY BODY Y | True vertical speed, relative to aircraft axis |
| VELOCITY WORLD Z | Speed relative to earth, in North/South direction |
| VELOCITY WORLD X | Speed relative to earth, in East/West direction |
| VELOCITY WORLD Y | Speed relative to earth, in vertical direction |
| RELATIVE WIND VELOCITY BODY X | Lateral speed relative to wind |
| RELATIVE WIND VELOCITY BODY Y | Vertical speed relative to wind |
| RELATIVE WIND VELOCITY BODY Z | Longitudinal speed relative to wind |
| PLANE ALT ABOVE GROUND | Altitude above the surface |
| PLANE LATITUDE | Self Explanatory |
| PLANE LONGITUDE | Self Explanatory |
| PLANE ALTITUDE | Self Explanatory |
| PLANE PITCH DEGREES | Self Explanatory |
| PLANE BANK DEGREES | Self Explanatory |
| PLANE HEADING DEGREES TRUE | Heading relative to true north |
| PLANE HEADING DEGREES MAGNETIC | Heading relative to magnetic north |
| MAGVAR | Magnetic variation |
| GROUND ALTITUDE | Altitude of surface |
| SURFACE TYPE | Type of surface (enum: 0 Concrete 1 Grass 2 Water 3 Grass_bumpy 4 Asphalt 5 Short_grass 6 Long_grass 7 Hard_turf 8 Snow 9 Ice 10 Urban 11 Forest 12 Dirt 13 Coral 14 Gravel 15 Oil_treated 16 Steel_mats 17 Bituminus 18 Brick 19 Macadam 20 Planks 21 Sand 22 Shale 23 Tarmac 24 Wright_flyer_track ) |
| SIM ON GROUND | On ground flag (bool) |
| INCIDENCE ALPHA | Angle of attack |
| INCIDENCE BETA | Sideslip angle |

Flight Instrumentation Data

| Simulation Variable | Description |
| --- | --- |
| AIRSPEED TRUE | True airspeed |
| AIRSPEED INDICATED | Indicated airspeed |
| AIRSPEED TRUE CALIBRATE | Angle of “True” calibration scale on airspeed indicator |
| AIRSPEED BARBER POLE | Redline airspeed (dynamic on some aircraft) |
| AIRSPEED MACH | Current mach |
| VERTICAL SPEED | Vertical speed indication |
| MACH MAX OPERATE | Maximum design mach |
| STALL WARNING | Stall warning state (bool) |
| OVERSPEED WARNING | Overspeed warning state (bool) |
| BARBER POLE MACH | Mach associated with maximum airspeed |
| INDICATED ALTITUDE | Altimeter indication |
| KOHLSMAN SETTING MB | Altimeter setting |
| KOHLSMAN SETTING HG | Altimeter setting |
| ATTITUDE INDICATOR PITCH DEGREES | AI pitch indication |
| ATTITUDE INDICATOR BANK DEGREES | AI bank indication |
| ATTITUDE BARS POSITION | AI reference pitch reference bars |
| ATTITUDE CAGE | AI caged state (bool) |
| WISKEY COMPASS INDICATION DEGREES | Magnetic compass indication |
| PLANE HEADING DEGREES GYRO | Heading indicator (directional gyro) indication |
| HEADING INDICATOR | Heading indicator (directional gyro) indication |
| GYRO DRIFT ERROR | Angular error of heading indicator |
| DELTA HEADING RATE | Rate of turn of heading indicator |
| TURN COORDINATOR BALL | Percent of ball deflection |
| ANGLE OF ATTACK INDICATOR | AoA indication |
| RADIO HEIGHT | Radar altitude |
| PARTIAL PANEL ADF | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL AIRSPEED | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL ALTIMETER | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL ATTITUDE | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL COMM | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL COMPASS | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL ELECTRICAL | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL AVIONICS | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL ENGINE | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL FUEL INDICATOR | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL HEADING | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL VERTICAL VELOCITY | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL TRANSPONDER | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL NAV | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL PITOT | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL TURN COORDINATOR | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| PARTIAL PANEL VACUUM | Gauge fail flag (0=ok, 1=fail, 2=blank) |
| MAX G FORCE | Maximum G force attained |
| MIN G FORCE | Minimum G force attained |
| SUCTION PRESSURE | Vacuum system suction pressure |

Avionics Data

Note:  Avionics parameters take an index (e.g., 1 or 2) for Nav 1, Nav 2, Com 1, Com 2, ADF 1, ADF 2, etc.

| Simulation Variable | Description |
| --- | --- |
| AVIONICS MASTER SWITCH | Avionics switch state (bool) |
| COM TRANSMIT | Audio panel com transmit state (bool) |
| COM RECIEVE ALL | Flag if all Coms receiving (bool) |
| NAV SOUND | Nav audio flag (bool) |
| DME SOUND | DME audio flag (bool) |
| ADF SOUND | ADF audio flag (bool) |
| MARKER SOUND | Marker audio flag (bool) |
| COM ACTIVE FREQUENCY | Com frequency |
| COM STANDBY FREQUENCY | Com standby frequency |
| COM STATUS | Radio status flag (enum: -1=Invalid 0 =Ok 1 = Does not exist 2 = No Electric 3 = Failed) |
| NAV AVAILABLE | Flag if Nav equipped on aircraft (bool) |
| NAV ACTIVE FREQUENCY | Nav active frequency |
| NAV STANDBY FREQUENCY | Nav standby frequency |
| NAV SIGNAL | Nav signal strength |
| NAV HAS NAV | Flag if Nav has signal (bool) |
| NAV HAS LOCALIZER | Flag if tuned station is a localizer (bool) |
| NAV HAS DME | Flag if tuned station has a DME (bool) |
| NAV HAS GLIDE SLOPE | Flag if tuned station has a glideslope (bool) |
| NAV BACK COURSE FLAGS | Flag if tuned to a back course localizer (bool) |
| NAV MAGVAR | Magnetic variation of tuned nav station |
| NAV RADIAL | Radial that aircraft is on |
| NAV RADIAL ERROR | Difference between current radial and OBS tuned radial |
| NAV LOCALIZER | Localizer course heading |
| NAV GLIDE SLOPE ERROR | Difference between current position and glideslope angle |
| NAV CDI | CDI needle deflection (+/- 127) |
| NAV GSI | Glideslope needle deflection (+/- 119) |
| NAV TOFROM | Nav TO/FROM flag (enum: 0=Off, 1=TO, 2=FROM) |
| NAV GS FLAG | Glideslope flag (bool) |
| NAV OBS | OBS setting |
| NAV DME | DME distance |
| NAV DMESPEED | DME speed |
| ADF ACTIVE FREQUENCY | ADF frequency |
| ADF STANDBY FREQUENCY | ADF standby frequency |
| ADF RADIAL | Current direction from NDB station |
| ADF SIGNAL | Signal strength |
| TRANSPONDER CODE | 4-digit code (use units bco16) |
| MARKER BEACON STATE | Marker beacon state (enum: 0=None, 1=Outer, 2=Middle, 3=Inner) |
| INNER MARKER | Inner marker state (bool) |
| MIDDLE MARKER | Middle marker state (bool) |
| OUTER MARKER | Outer marker state (bool) |
| NAV RAW GLIDE SLOPE | Glide slope angle |
| ADF CARD | ADF compass rose setting |
| HSI CDI NEEDLE | Needle deflection (+/- 127) |
| HSI GSI NEEDLE | Needle deflection (+/- 119) |
| HSI CDI NEEDLE VALID | Signal valid (bool) |
| HSI GSI NEEDLE VALID | Signal valid (bool) |
| HSI TF FLAGS | Nav TO/FROM flag (enum: 0=Off, 1=TO, 2=FROM) |
| HSI BEARING | Bearing of needle |
| HSI BEARING VALID | Signal valid (bool) |
| HSI HAS LOCALIZER | Station is a localizer (bool) |
| HSI SPEED | DME/GPS speed |
| HSI DISTANCE | DME/GPS distance |
| GPS POSITION LAT | Current GPS latitude |
| GPS POSITION LON | Current GPS longitude |
| GPS POSITION ALT | Current GPS altitude |
| GPS MAGVAR | Current GPS magnetic variation |
| GPS IS ACTIVE FLIGHT PLAN | Flight plan mode active (bool) |
| GPS IS ACTIVE WAY POINT | Waypoint mode active (bool) |
| GPS IS ARRIVED | Is flight plan destination reached (bool) |
| GPS IS DIRECTTO FLIGHTPLAN | Is Direct To Waypoint mode active (bool) |
| GPS GROUND SPEED | Current ground speed |
| GPS GROUND TRUE HEADING | Current true heading |
| GPS GROUND MAGNETIC TRACK | Current magnetic ground track |
| GPS GROUND TRUE TRACK | Current true ground track |
| GPS WP DISTANCE | Distance to waypoint |
| GPS WP BEARING | Magnetic bearing to waypoint |
| GPS WP TRUE BEARING | True bearing to waypoint |
| GPS WP CROSS TRK | Cross track distance |
| GPS WP DESIRED TRACK | Desired track to waypoint |
| GPS WP TRUE REQ HDG | Required true heading to waypoint |
| GPS WP VERTICAL SPEED | Vertical speed to waypoint |
| GPS WP TRACK ANGLE ERROR | Tracking angle error to waypoint |
| GPS ETE | Estimated time enroute to destination |
| GPS ETA | Estimated time of arrival at destination |
| GPS WP NEXT LAT | Latitude of next waypoint |
| GPS WP NEXT LON | Longitude of next waypoint |
| GPS WP NEXT ALT | Altitude of next waypoint |
| GPS WP PREV VALID | Is previous waypoint valid (i.e. current waypoint is not the first waypoint) (bool) |
| GPS WP PREV LAT | Latitude of previous waypoint |
| GPS WP PREV LON | Longitude of previous waypoint |
| GPS WP PREV ALT | Altitude of previous waypoint |
| GPS WP ETE | Estimated time enroute to waypoint |
| GPS WP ETA | Estimated time of arrival at waypoint |
| GPS COURSE TO STEER | Suggested heading to steer (for autopilot) |
| GPS FLIGHT PLAN WP INDEX | Index of waypoint |
| GPS FLIGHT PLAN WP COUNT | Number of waypoints |
| GPS IS ACTIVE WP LOCKED | Is switching to next waypoint locked (bool) |
| GPS IS APPROACH LOADED | Is approach loaded (bool) |
| GPS IS APPROACH ACTIVE | Is approach mode active (bool) |
| GPS APPROACH MODE | Sub mode within approach mode (enum: 0=None 1=Transition 2=Final 3=Missed) |
| GPS APPROACH WP TYPE | Waypoint type within approach mode (enum: 0=None 1=Fix 2=Procedure turn left 3=Procedure turn right 4=Dme arc left 5=Dme arc right 6=Holding left 7=Holding right 8=Distance 9=Altitude 10=Manual sequence 11= Vectors to final) |
| GPS APPROACH IS WP RUNWAY | Waypoint is the runway |
| GPS APPROACH SEGMENT TYPE | Seqment type within approach (enum: 0=Line 1=Arc clockwise 2=Arc counter-clockwise) |
| GPS APPROACH APPROACH INDEX | Index of approach for given airport |
| GPS APPROACH APPROACH TYPE | Approach type (enum: 0=None 1=GPS 2=VOR 3=NDB 4=ILS 5=Localizer 6=SDF 7=LDA 8=VOR/DME 9=NDB/DME 10=RNAV 11=Backcourse) |
| GPS APPROACH TRANSITION INDEX | Index of approach transition |
| GPS APPROACH IS FINAL | Is approach transition final approach segment (bool) |
| GPS APPROACH IS MISSED | Is approach segment missed approach segment (bool) |
| GPS APPROACH TIMEZONE DEVIATION | Deviation of local time from GMT |
| GPS APPROACH WP INDEX | Index of current waypoint |
| GPS APPROACH WP COUNT | Number of waypoints |
| GPS DRIVES NAV1 | GPS is driving Nav 1 indicator (bool) |

Controls Data

| Simulation Variable | Description |
| --- | --- |
| YOKE Y POSITION | Percent control deflection fore/aft (for animation) |
| YOKE X POSITION | Percent control deflection left/right (for animation) |
| RUDDER PEDAL POSITION | Percent rudder pedal deflection (for animation) |
| RUDDER POSITION | Percent rudder input deflection |
| ELEVATOR POSITION | Percent elevator input deflection |
| AILERON POSITION | Percent aileron input left/right |
| ELEVATOR TRIM POSITION | Elevator trim deflection |
| ELEVATOR TRIM INDICATOR | Percent elevator trim (for indication) |
| ELEVATOR TRIM PCT | Percent elevator trim |
| BRAKE LEFT POSITION | Percent left brake |
| BRAKE RIGHT POSITION | Percent right brake |
| BRAKE INDICATOR | Brake on indication |
| BRAKE PARKING POSITION | Parking brake on (bool) |
| BRAKE PARKING INDICATOR | Parking brake indicator (bool) |
| SPOILERS ARMED | Auto-spoilers armed (bool) |
| SPOILERS HANDLE POSITION | Spoiler handle position |
| SPOILERS LEFT POSITION | Percent left spoiler deflected |
| SPOILERS RIGHT POSITION | Percent right spoiler deflected |
| FLAPS HANDLE PERCENT | Percent flap handle extended |
| FLAPS HANDLE INDEX | Index of current flap position |
| FLAPS NUM HANDLE POSITIONS | Number of flap positions |
| TRAILING EDGE FLAPS LEFT PERCENT | Percent left trailing edge flap extended |
| TRAILING EDGE FLAPS RIGHT PERCENT | Percent right trailing edge flap extended |
| TRAILING EDGE FLAPS LEFT ANGLE | Angle left trailing edge flap extended |
| TRAILING EDGE FLAPS RIGHT ANGLE | Angle right trailing edge flap extended |
| LEADING EDGE FLAPS LEFT PERCENT | Percent left leading edge flap extended |
| LEADING EDGE FLAPS RIGHT PERCENT | Percent right leading edge flap extended |
| LEADING EDGE FLAPS LEFT ANGLE | Angle left leading edge flap extended |
| LEADING EDGE FLAPS RIGHT ANGLE | Angle right leading edge flap extended |
| IS GEAR RETRACTABLE | Self explanatory (bool) |
| IS GEAR SKIS | Self explanatory (bool) |
| IS GEAR FLOATS | Self explanatory (bool) |
| IS GEAR SKIDS | Self explanatory (bool) |
| IS GEAR WHEELS | Self explanatory (bool) |
| GEAR HANDLE POSITION | Self explanatory |
| GEAR HYDRAULIC PRESSURE | Self explanatory |
| TAILWHEEL LOCK ON | Self explanatory (bool) |
| GEAR CENTER POSITION | Percent center gear extended |
| GEAR LEFT POSITION | Percent left gear extended |
| GEAR RIGHT POSITION | Percent right gear extended |
| GEAR TAIL POSITION | Percent tail gear extended |
| GEAR AUX POSITION | Percent auxiliary gear extended |
| GEAR POSITION | Percent gear extended (takes index) |
| GEAR ANIMATION POSITION | Percent gear animation extended (takes index) |
| GEAR TOTAL PCT EXTENDED | Percent total gear extended |
| AUTO BRAKE SWITCH CB | Auto brake switch position |
| WATER RUDDER HANDLE POSITION | Self explanatory |
| ELEVATOR DEFLECTION | Angle deflection |
| ELEVATOR DEFLECTION PCT | Percent deflection |
| WATER LEFT RUDDER EXTENDED | Percent extended |
| WATER RIGHT RUDDER EXTENDED | Percent extended |
| GEAR CENTER STEER ANGLE | Self explanatory |
| GEAR LEFT STEER ANGLE | Self explanatory |
| GEAR RIGHT STEER ANGLE | Self explanatory |
| GEAR AUX STEER ANGLE | Self explanatory |
| GEAR STEER ANGLE | Self explanatory (takes index) |
| WATER LEFT RUDDER STEER ANGLE | Self explanatory |
| WATER RIGHT RUDDER STEER ANGLE | Self explanatory |
| GEAR CENTER STEER ANGLE PCT | Self explanatory |
| GEAR LEFT STEER ANGLE PCT | Self explanatory |
| GEAR RIGHT STEER ANGLE PCT | Self explanatory |
| GEAR AUX STEER ANGLE PCT | Self explanatory |
| GEAR STEER ANGLE PCT | Self explanatory (takes index) |
| WATER LEFT RUDDER STEER ANGLE PCT | Self explanatory |
| WATER RIGHT RUDDER STEER ANGLE PCT | Self explanatory |
| AILERON LEFT DEFLECTION | Angle deflection |
| AILERON LEFT DEFLECTION PCT | Percent deflection |
| AILERON RIGHT DEFLECTION | Angle deflection |
| AILERON RIGHT DEFLECTION PCT | Percent deflection |
| AILERON AVERAGE DEFLECTION | Angle deflection |
| AILERON TRIM | Angle deflection |
| AILERON TRIM PCT | Percent deflection |
| RUDDER DEFLECTION | Angle deflection |
| RUDDER DEFLECTION PCT | Percent deflection |
| RUDDER TRIM | Angle deflection |
| RUDDER TRIM PCT | Percent deflection |
| FLAPS AVAILABLE | Self explanatory (bool) |
| GEAR DAMAGE BY SPEED | Self explanatory (bool) |
| GEAR SPEED EXCEEDED | Self explanatory (bool) |
| FLAP DAMAGE BY SPEED | Self explanatory (bool) |
| FLAP SPEED EXCEEDED | Self explanatory (bool) |
| CENTER WHEEL RPM | Self explanatory |
| LEFT WHEEL RPM | Self explanatory |
| RIGHT WHEEL RPM | Self explanatory |

Autopilot Data

| Simulation Variable | Description |
| --- | --- |
| AUTOPILOT AVAILABLE | Available flag (bool) |
| AUTOPILOT MASTER | On/off flag (bool) |
| AUTOPILOT NAV SELECTED | Index of Nav radio selected |
| AUTOPILOT WING LEVELER | Wing leveler active (bool) |
| AUTOPILOT NAV1 LOCK | Lateral nav mode active (bool) |
| AUTOPILOT HEADING LOCK | Heading mode active (bool) |
| AUTOPILOT HEADING LOCK DIR | Selected heading |
| AUTOPILOT ALTITUDE LOCK | Altitude hole active (bool) |
| AUTOPILOT ALTITUDE LOCK VAR | Selected altitude |
| AUTOPILOT ATTITUDE HOLD | Attitude hold active (bool) |
| AUTOPILOT GLIDESLOPE HOLD | GS hold active (bool) |
| AUTOPILOT PITCH HOLD REF | Current reference pitch |
| AUTOPILOT APPROACH HOLD | Approach mode active (bool) |
| AUTOPILOT BACKCOURSE HOLD | Back course mode active (bool) |
| AUTOPILOT VERTICAL HOLD VAR | Selected vertical speed |
| AUTOPILOT FLIGHT DIRECTOR ACTIVE | Flight director active (bool) |
| AUTOPILOT FLIGHT DIRECTOR PITCH | Reference pitch angle |
| AUTOPILOT FLIGHT DIRECTOR BANK | Reference bank angle |
| AUTOPILOT AIRSPEED HOLD | Airspeed hold active (bool) |
| AUTOPILOT AIRSPEED HOLD VAR | Selected airspeed |
| AUTOPILOT MACH HOLD | Mach hold active (bool) |
| AUTOPILOT MACH HOLD VAR | Selected mach |
| AUTOPILOT YAW DAMPER | Yaw damper active (bool) |
| AUTOPILOT RPM HOLD VAR | Selected rpm |
| AUTOPILOT THROTTLE ARM | Auto–throttle armed (bool) |
| AUTOPILOT TAKEOFF POWER ACTIVE | Takeoff / Go Around power mode active |
| AUTOTHROTTLE ACTIVE | Auto-throttle active (bool) |

Environment Data

| Simulation Variable | Description |
| --- | --- |
| AMBIENT DENSITY | Self Explanatory |
| AMBIENT TEMPERATURE | Self Explanatory |
| AMBIENT PRESSURE | Self Explanatory |
| AMBIENT WIND VELOCITY | Self Explanatory |
| AMBIENT WIND DIRECTION | Self Explanatory |
| AMBIENT WIND X | Wind component in East/West |
| AMBIENT WIND Y | Wind component in vertical direction |
| AMBIENT WIND Z | Wind component in North/South direction |
| AMBIENT PRECIP STATE | Precip state (bit field) |
| AIRCRAFT WIND X | Wind component in aircraft lateral axis |
| AIRCRAFT WIND Y | Wind component in aircraft vertical axis |
| AIRCRAFT WIND Z | Wind component in aircraft longitudinal axis |
| BAROMETER PRESSURE | Self Explanatory |
| SEA LEVEL PRESSURE | Self Explanatory |
| TOTAL AIR TEMPERATURE | Self Explanatory |
| WINDSHIELD RAIN EFFECT AVAILABLE | Is visual effect available on this aircraft (bool) |

Helicopter Specific Data

Note: Currently only available on the R22 in FS9.

| Simulation Variable | Description |
| --- | --- |
| ROTOR BRAKE HANDLE POS | Percent actuated |
| ROTOR BRAKE ACTIVE | Active (bool) |
| ROTOR CLUTCH SWITCH POS | Switch position (bool) |
| ROTOR CLUTCH ACTIVE | Active (bool) |
| ROTOR TEMPERATURE | Main rotor transmission temperature |
| ROTOR CHIP DETECTED | Chip detection (bool) |
| ROTOR GOV SWITCH POS | Switch position (bool) |
| ROTOR GOV ACTIVE | Active (bool) |
| ROTOR LATERAL TRIM PCT | Trim percent |
| ROTOR RPM PCT | Percent max rated rpm |

Miscellaneous Systems Data

| Simulation Variable | Description |
| --- | --- |
| SMOKE ENABLE | Smoke system active (bool) |
| SMOKESYSTEM AVAILABLE | Smoke system available (bool) |
| PITOT HEAT | Pitot heat active (bool) |
| FOLDING WING LEFT PERCENT | Self explanatory |
| FOLDING WING RIGHT PERCENT | Self explanatory |
| CANOPY OPEN | Percent primary door/exit open |
| TAILHOOK POSITION | Percent tail hook extended |
| EXIT OPEN | Percent door/exit open (takes index) |
| STALL HORN AVAILABLE | Self explanatory (bool) |
| ENGINE MIXURE AVAILABLE | Self explanatory (bool) |
| CARB HEAT AVAILABLE | Self explanatory (bool) |
| SPOILER AVAILABLE | Self explanatory (bool) |
| IS TAIL DRAGGER | Self explanatory (bool) |
| STROBES AVAILABLE | Self explanatory (bool) |
| TOE BRAKES AVAILABLE | Self explanatory (bool) |
| PUSHBACK STATE | Type of pushback (enum: 0=Straight, 1=Left, 2=Right) |
| ELECTRICAL MASTER BATTERY | Battery switch position (bool) |
| ELECTRICAL TOTAL LOAD AMPS | Self explanatory |
| ELECTRICAL BATTERY LOAD | Self explanatory |
| ELECTRICAL BATTERY VOLTAGE | Self explanatory |
| ELECTRICAL MAIN BUS VOLTAGE | Self explanatory |
| ELECTRICAL MAIN BUS AMPS | Self explanatory |
| ELECTRICAL AVIONICS BUS VOLTAGE | Self explanatory |
| ELECTRICAL AVIONICS BUS AMPS | Self explanatory |
| ELECTRICAL HOT BATTERY BUS VOLTAGE | Self explanatory |
| ELECTRICAL HOT BATTERY BUS AMPS | Self explanatory |
| ELECTRICAL BATTERY BUS VOLTAGE | Self explanatory |
| ELECTRICAL BATTERY BUS AMPS | Self explanatory |
| ELECTRICAL GENALT BUS VOLTAGE | Self explanatory (takes engine index) |
| ELECTRICAL GENALT BUS AMPS | Self explanatory (takes engine index) |
| CIRCUIT GENERAL PANEL ON | Is electrical power available to this circuit (bool) |
| CIRCUIT FLAP MOTOR ON | Is electrical power available to this circuit (bool) |
| CIRCUIT GEAR MOTOR ON | Is electrical power available to this circuit (bool) |
| CIRCUIT AUTOPILOT ON | Is electrical power available to this circuit (bool) |
| CIRCUIT AVIONICS ON | Is electrical power available to this circuit (bool) |
| CIRCUIT PITOT HEAT ON | Is electrical power available to this circuit (bool) |
| CIRCUIT PROP SYNC ON | Is electrical power available to this circuit (bool) |
| CIRCUIT AUTO FEATHER ON | Is electrical power available to this circuit (bool) |
| CIRCUIT AUTO BRAKES ON | Is electrical power available to this circuit (bool) |
| CIRCUIT STANDY VACUUM ON | Is electrical power available to this circuit (bool) |
| CIRCUIT MARKER BEACON ON | Is electrical power available to this circuit (bool) |
| CIRCUIT GEAR WARNING ON | Is electrical power available to this circuit (bool) |
| CIRCUIT HYDRAULIC PUMP ON | Is electrical power available to this circuit (bool) |
| HYDRAULIC PRESSURE | Self explanatory |
| HYDRAULIC RESERVOIR PERCENT | Self explanatory |
| HYDRAULIC SYSTEM INTEGRITY | Percent system functional |
| STRUCTURAL DEICE SWITCH | Self explanatory |

Miscellaneous Data

| Simulation Variable | Description |
| --- | --- |
| TOTAL WEIGHT | Self explanatory |
| MAX GROSS WEIGHT | Self explanatory |
| EMPTY WEIGHT | Self explanatory |
| IS USER SIM | Is this the user loaded aircraft (bool) |
| SIM DISABLED | Is sim disabled (bool) |
| G FORCE | Self explanatory |
| ATC HEAVY | Is this aircraft recognized by ATC as “heavy” (bool) |
| AUTO COORDINATION | Is auto-coordination active (bool) |
| REALISM | General realism percent |
| DESIGN SPEED VS0 | Self explanatory |
| DESIGN SPEED VS1 | Self explanatory |
| DESIGN SPEED VC | Self explanatory |
| MIN DRAG VELOCITY | Self explanatory |
| ESTIMATED CRUISE SPEED | Self explanatory |
| CG PERCENT | Longitudinal CG position as a percent of reference chord |
| CG PERCENT LATERAL | Lateral CG position as a percent of reference chord |
| IS SLEW ACTIVE | Self explanatory |
| IS SLEW ALLOWED | Self explanatory |
| ATC SUGGESTED MIN RWY TAKEOFF | Suggested minimum runway length for takeoff. Used by ATC |
| ATC SUGGESTED MIN RWY LANDING | Suggested minimum runway length for landing. Used by ATC |
| PAYLOAD STATION WEIGHT | Individual payload station weight (takes index) |
| PAYLOAD STATION COUNT | Number of payload stations |
| USER INPUT ENABLED | Is input allowed from the user (bool) |
| TYPICAL DESCENT RATE | Self explanatory |
| VISUAL MODEL RADIUS | Self explanatory |
| CATEGORY | Aircraft category (enum: 0=Airplane, 1=Helicopter, 2=Airplane – AI controlled only) |

Aircraft String Data

| Simulation Variable | Description |
| --- | --- |
| ATC TYPE | “Type” used by ATC |
| ATC MODEL | “Model” used by ATC |
| ATC ID | “ID” used by ATC |
| ATC AIRLINE | “Airline” used by ATC |
| ATC FLIGHT_NUMBER | “Flight Number” used by ATC |
| TITLE | “Title” from aircraft.cfg |
| HSI STATION IDENT | Tuned station identifier |
| GPS WP_PREV ID | ID of previous GPS waypoint |
| GPS WP_NEXT ID | ID of next GPS waypoint |
| GPS APPROACH AIRPORT ID | ID of airport |
| GPS APPROACH APPROACH ID | ID of approach |
| GPS APPROACH TRANSITION ID | ID of approach transition |

Simulator Global Data

| Simulation Variable | Description |
| --- | --- |
| ABSOLUTE TIME | Time, as referenced from 12:00 AM January 1, 0000 |
| ZULU TIME | Greenwich Mean Time (GMT) |
| ZULU DAY OF WEEK | GMT day of week |
| ZULU DAY OF MONTH | GMT day of month |
| ZULU MONTH OF YEAR | GMT month of year |
| ZULU DAY OF YEAR | GMT day of year |
| ZULU YEAR | GMT year |
| LOCAL TIME | Local time |
| LOCAL DAY OF WEEK | Local day of week |
| LOCAL DAY OF MONTH | Local day of month |
| LOCAL MONTH OF YEAR | Local month of year |
| LOCAL DAY OF YEAR | Local day of year |
| LOCAL YEAR | Local year |
| TIME ZONE OFFSET | Local time difference from GMT |
| TIME OF DAY | General time of day (enum: 1=Day, 2=Dusk/Dawn, 3=Night) |
| SIMULATION RATE | Time acceleration factor |
| UNITS OF MEASURE | Units of measure (enum: 0=English, 1=Metric (with altitude in feet), 2=Metric (with altitude in meters) ) |